Gauge 304

Calibration Date: 25 April 2009

Installation Date: 10 June 2009, ~11:30 am

Big Cataloochee Mountain (35˚40.206’ N, 83˚10.972’ W, ~5971 ft)

Way Point 142

Visits:

10 June 2009 (Installation)

Mostly cloudy

24 October 2009 (~1300)

Partly cloudy, calm. Had to replace sign. Leaves and debris cleared from gauge top. Bounce sheet added. No releveling necessary. Most leaves in area had already fallen. Data download, full record, both files okay. Time adjusted by 2 minutes. Tipping bucket about half full.

6 June 2010 (~8:30 am)

Fog and windy. Datta logger loose/fixed with tape, full data record, replace Bounce sheet, time good, bucket ¾ full, clean minor debris from funnel, slight re-level, clear vegetation.