Gauge 100-T

Calibration Date: 30 January 2009

Serial #:

Date Installed: 31 January 2009, ~1300

Purchase Knob research station, at old tennis court (~4905 ft)

About 5-7 m away from: (when your back is to the house, it is the leftmost gauge.)

Way Point 42

35°35.166’, 83°04.352’

Gauge installed for comparison, to try and determine the best method to control debris in the gauge top, and to define experimental uncertainties.

Visits:

31 January 2009 (~1300)

Installed. Clear skies, breezy, some low cumulus to the north (behind you if the house is on your right.)

Time set to summer time (1 hour off, to match Gauge 100)

15 May 2009 (?)

Taken out of field for recalibration

22 May 2009

Recalibrated

1 June 2009 (~1400)

Affixed new top to gauge (first method attempted to reduce debris collection in the gauge top). Reinstalled.

Partly cloudy.

24 June 2009 (~1300)

Uploaded data, did not clear, date/time correct. No releveling necessary. Cleaned out gauge top – lots of bird droppings. Clear skies overhead.

26 August 2009 (~1500)

Took gauge from where it was stationed in Maggie Valley (since mid-July) and re-installed it next to the met tower at Purchase Knob. Data downloaded, date and time correct, Bounce sheet added, top cleared of debris.

8 November 2009 (~1130)

Downloaded data, full record. Date/time correct (for summer time). No releveling necessary. Very minor cleaning done to gauge top – Bounce sheet and sign were both in place. Ladybugs cleared out from beneath the Hydrological Services cap.

22 May 2010 (~10 am)

Spiderwebs and some debris cleaned from gauge top. No releveling necessary. Bounce sheet in place. Downloaded data, did not clear, full record. Sunny – rain earlier in the day, some water in bucket.

Directions:

40W to Exit 20 (276)

Merge on 276. (~3 miles)

Right on Grindstone Road (~1 mile)

Right on Hemphill Road (~3 mile) Continue up the gravel road. If during business hours, gate should be open. Be sure that you find out the code just in case, it changes. Drive up to the house. You can see the rain gauge down the hill from the deck – at the old tennis court. Contact there is Paul Super.