Gauge 1

Calibration Date: 6/24,6/27/07

Date Installed: June 30, 2007

Camp Daniel Boone (~3794 feet)

Near Deep Gap (35.23’53.9, 82.54’46.9)

Visits:

October 16, 2007 12:00 pm

Replaced all 6 batteries of data logger. Changed data logger from 00:00:24 to 12:06:00.

March 5, 2008

Gauge looks to be in good condition – full data record for HOBO – old logger end 14 Feb 08 (batteries dead?) – time on old logger 13:59 EDT –

AAs changed on old logger – battery changed on HOBO

June 4, 2008

Gauge has some dents but is upright and needs only slight leveling.

Full record for HOBO

Could not download old logger – took it with us as we had no batteries to attempt replacement. It later turned out to be nonfunctional, although we kept the cable which works.

Calibration began at 10:25 am. Not recorded by HOBO. Tested HOBO again before leaving and it seemed to be working with test tips.

June 9, 2008

HOBO may or may not have worked during week. No tips recorded. Had trouble again getting it to register tests. Eventually it did successfully record our calibration data.

Old, nonfunctional logger replaced with the one from Gauge 7. New batteries and time changed.

Calibration of 100 mm and 300 mm nozzles successfully completed. We had some thunder and light showers but rigged an umbrella over the tube, and feel that this was successful since there was hardly any wind.

June 11, 2008

1 tip recorded for field data on 6/10/08 before calibration began.

Calibration completed for 200 mm and 50 mm nozzles. Sunny and warm

all afternoon.

At gauge from approx 2:15-6:15 pm.

October 25, 2008

Gauge looks good – needed slight releveling. Vegetation was clear around 			gauge.

Datalogger and HOBO batteries changed and data downloaded. Both

Loggers had the full data record.  Debris cleaned out of gauge top.  Weather – sunny.

March 21, 2009

Gauge upright – vegetation clear.

Weather was sunny, clear, and calm.

Full data record on both HOBO and old logger.

Old logger’s time off by ~10 min, fixed. Also changed AAs.

Minor releveling completed – cleaned out gauge top.

October 10, 2009

~1:30 pm

Light rain. Gauge upright, slight releveling necessary. Added bounce sheet, insecticide – sign in place. Cleaned out gauge top. Replaced old logger with new- chanbe time to EDT. Changed battery on HOBO – downloaded data-full record. Full record on old logger.

May 1, 2010

~9:30 am

Gauge was upright, but slight releveling was necessary. The sign was still in place. We replaced the Bounce sheet, added insecticide and cleaned out the gauge top. The forest across the road from the gauge location appeared to have undergone a recent controlled burn – forest floor only. We downloaded the new datalogger and had to adjust the time 3 minutes. We changed the battery on the HOBO, did test tips and relaunched. The full data record was available for both loggers.

Directions:

40W to Exit 31 (215)

215S (Stay on this road through several turns).

Left on SR 1129.

On right, just past a bridge, you will see a road with a marker in front that says “2370”. Turn up this road and you will see the gate on your left just before a barn.  After the gate that we have a key for, keep following the road until you see another orange gate up a small hill to the right. Then follow that road until you see the gauge in a clearing on the left (at an intersection with another path).

Contact: Eddie Hollifield (648-6770)

We have gate key for this gauge.