Gauge 100

Calibration Date: 5/21/08

Serial Number 753

Date Installed:	 30 May 2008 ~1600

Purchase Knob Research Station, at old tennis court (~4905 ft)

Way Point 42

35°35.166’, 83°04.352’

Visits:

30 May 2008 (Installation)

Building cumulus clouds & lots of insects out.

31 July 2008 (~0900)

Uploaded and cleared data.

Record was complete, date and time correct.

4/8 cumulus clouds.

26 November 2008 (day before Thanksgiving, ~1400)

Uploaded data. Did not clear.

Cleaned out top of rain gauge, tiny amount of debris.

Clear weather.

31 January 2009 (~1100)

Uploaded data. Did not clear.

Record complete. Cleaned out gauge top (small amount of debris)

Weather clear, breezy, some low cumulus to the north (behind you if the house is to your right.)

Time on gauge is one hour off (12:11) did not change.

Block of ice in one side of the tipping bucket. Did not remove.

1 June 2009 (~1400 local)

Uploaded and cleared data. Cleaned minor debris from gauge top – washer 			was broken.

Current weather – partly cloudy.

24 June 2009 (~1300)

Uploaded and did not clear data. Date/time correct. No releveling necessary. Cleaned out gauge top – lots of bird droppings. Clear skies overhead.

26 August 2009 (~1600)

Uploaded, did not clear data. Date/time correct. No releveling necessary. Cleaned out gauge top, lots of bird droppings, small debris, a dead bumblebee. Bounce sheet added, base of post treated with insecticide. Replaced broken washer.

8 November 2009 (noon)

No releveling necessary. Spider webs cleaned from beneath the Hydrological Services cap. Minor debris and bird droppings cleared from gauge top. Bounce sheet and sign in place. Data downloaded, full record, date and time correct.

May 22, 2010 (~10 am)

Lots of debris in gauge top: family of spiders, bird droppings, dirt. Gauge needed minor releveling. Cleaned out top, replaced Bounce sheet. Date/time correct. Data download – not cleared – full record. Sunny, rained earlier in the day.

Directions:

40W to Exit 20 (276)

Merge on 276. (~3 miles)

Right on Grindstone Road (~1 mile)

Right on Hemphill Road (~3 mile) Continue up the gravel road. If during business hours, gate should be open. Be sure that you find out the code just in case, it changes. Drive up to the house. You can see the rain gauge down the hill from the deck – at the old tennis court. Contact there is Paul Super.