Gauge 8

Calibration Date: 8/7/07, 8/22/07

Date Installed: 8/24/07

Near Double Spring Gap (~5700 ft)

Way Point 25, Site 5

35.22.927’, 82.58.414’

Visits:

December 19, 2007

11:02 am EST

Data downloaded & old logger time changed to local time (EST).

One stake loose and dents found on gauge case.

Replaced AAs on old logger.

April 4, 2008

Gauge tilted but still upright.

HOBO inop, taken back to UNCA

Changed AAs on datalogger.

Left at 3:30 pm.

May 20, 2008

9:00 AM EST

Gauge was slightly tilted – we fixed it.

Original HOBO placed back with rain gauge, but still did not work.

Replaced HOBO with HOBO from Gauge 7, it worked.

Tested HOBO and relaunched. Replaced battery.

Old logger 1 hour behind on time.

July 14, 2008

~9:30 AM

Gauge was nearly level.

A plant was growing over the top of gauge (picture taken).

Got HOBO and backup logger data (full record).

Changed time to correct time on logger, changed AA batteries and

Cleared brush.

November 16, 2008 ~0900 local time

Gauge tipped completely over – all stakes up and bolts loosened.

Gauge reinstalled and leveled.

Batteries changed on both HOBO and old logger. Ice on old logger.

Records through 8/28 on both loggers.

March 27, 2009 ~0830 local time

Full data record on both loggers. AAs changed in old logger.

Time was 15 min. slow on old logger – fixed.

Weather overcast.

Minor releveling done, minor debris cleared from gauge top.

23 June 2009 (~1000)

Gauge was tipped over and two stakes pulled up. Data was collected until 19 June on both loggers. Batteries were changed on both loggers. Date and time were correct. The gauge was set up again and debris cleared from the top. The gauge base was treated with insecticide.

October 2, 2009

~9:45 am

Gauge was tipped over, so had to move gauge to 35.22.916’ N, 82.58.447’ W and place new cords. Old data logger unresponsive #KOPRUNG. Replaced HOBO with new one. Animal chew marks evident.

May 7, 2010

~9:57 am

Gauge upright – Daniel photos. Cleaned, slight re-level, add stakes, time okay, data good, HOBO battery replaced.

Directions:

At the Waynesville Watershed. Must contact Kyle and Hugh (watertreatment@townofwaynesville.org and parrot_l@bellsouth.net, respectively) to be escorted up to the gauges. Kyle’s number: 456-8497.

To the watershed: 40W to Exit 27 (74W)

74W to Exit 98 (West Waynesville)

Left off the exit (Hyatt Creek Road)

Left at S Main Street/US-23-BR

Right at Allens Creek Road

Follow the road right when you see the quarry in front of you. Make a left just after the quarry, and follow that road until you reach the watershed on your left. Park there and Hugh will take you up to the gauges.