Gauge 303

Calibration Date: 4/25/09

Installation Date: 5/19/09

Mt Cammerer (near lookout tower)

35°45.777’N, 83°09.733’W (~4887 ft)

Way Point 32

Vists:

Installation 19 May 2009 ~12:30 pm

Sunny, clear, calm.

21 November 2009

Wx: Sunny, clear, calm

Data download, no time adjustment necessary. Full record, did not clear. Sign missing (had no extras) Added bounce sheet. Slight re-leveling done, very minor debris cleared from gauge top.

26 May 2010

Mostly cloudy, full data downloaded (full record), add new Bounce sheet, clean minor debris(small dead bugs), no re-level, clear some vegetation