Gauge 104

Serial Number 751

Calibration Date: 5/22/08

ReCalibration with Gauge 7 Top: 8 August 2008

Installation Date: 6/17/08 (~5:30 pm)

Cataloochee Ski Area

35˚33.291’ N, 83˚05.280’ W

~5200 feet, Way Point 129

Visits:

17 June 2008 – Installation

Clear, sunny, pictures taken.

1 August 2008 ~ 12:30 pm

Gauge was knocked over – still on pole. Cement entirely out of the ground and sideways.  Pictures were taken.  There were dents in the gauge, including the top.

Data record goes through 19 June.

We reset the pole in a total of 100 pounds of cement and leveled it.

Date and time correct on logger.

4/8 cumulus cover

6 August 2008 ~1130 am

Gauge top torn off from base and tilted (pictures taken)

Took off base of gauge from location, top needs replacement.

21 August 2008 ~1000

Re-install gauge on ski-side of mountain. Partly cloudy – low chance of rain today.

24 June 2009 ~1100

Base of post treated with insecticide. Minor releveling done, debris cleared from gauge top. Data downloaded/not cleared/date and time correct. Clog below tipping bucket (water NOT high enough to touch bucket) removed. 2/8 cloud cover. Sign intact.

8 November 2009 (~1030)

No releveling necessary. Minor debris cleared from gauge top and inside gauge. Bounce sheet added. Old sign in place. Data download, full record. Kept summer time.

May 12, 2010 (~11 am – fog/drizzle)

No releveling necessary. Downloaded data, full record, changed Bounce sheet, time changed (off by one hour!)

Directions:

40W to Exit 27 (74W)

74W to Exit 103 (Maggie Valley – 19)

Follow 19 into Maggie Valley and make a right just past Ghost Town (Fie Top Road). (You will be in a right hand only lane.)

Follow that road up to Cataloochee Ranch/Ski Area. When you come to a T, make a left (follow sign to Ski Area)

Follow the fork in the road to the right (Sheep Pasture Road)

Follow Wagon Trail up the hill when the road forks again, to the left.

Left on Saddle Ridge.

Call Chris Bates with the Cataloochee Ski Area when going up, just to let him know. 926-0285 ext. 310.