Gauge 4

Calibration Date: 6/24/07, 7/18/07

(Confirmed use of Tube Duke #1)

Date Installed: 7/26/07

Lickstone Ridge, off Haywood-Jackson Overlook (~6305 ft)

Way Point 11, Site 3

35.22.097’, 82.59.410’

Visits:

October 20, 2007

Gauge tipped over – refastened and downloaded data

Data download complete at 12:14 pm EDT

HOBO says 17:17 when EDT is 12:17 pm, off by 1 hr

December 4, 2007

1:41 pm EST

Gauge upright and apparently untouched

Data looks good, 5 rain events since last visit

Changed time to local time

Replaced AA batteries on old logger

April 7, 2008

~11:30 am EDT

Old logger time EST

Replaced HOBO and old logger batteries

Data files complete (4 Dec 07 – 6 Apr 08)

Gauge in good shape

Re-leveled gauge using bubble-level

July 15, 2008

~10-10:30 AM local

Lots of plant life directly surrounding gauge, none over the top (pictures taken)

Full data record on both HOBO and datalogger

AAs changed on old logger

Cleared brush – no leveling necessary

December 19, 2008

~1-2 pm local

Cleared leaves from screen

Full data record on both loggers.

Year says “2007” on old logger datafile, should be “2008”

Slight re-level adjustment

Light rain during maintenance

April 18, 2009

~4 pm local

Gauge basically level – minor readjustments made.

Debris cleared from gauge top.

HOBO had tips from 12/19/08 only: changed battery, did test tips, relaunched.

Old logger had full record. AAs changed: after this there was some trouble with the old logger communicating with the rain gauge. We unplugged & replugged in the cable and test tips were successful.

Changed year on old logger (Was 2008 – now correct, 2009). Also time ~15 minutes slow, adjusted.

Weather: clear, sunny, calm.

18 July 2009, ~1400

Cleared debris from gauge top, cleared brush from area. Minor releveling done. New sign attached. Bounce sheet taped inside gauge. Full record on HOBO. Old logger has data up till 15 June 2009. Batteries changed on old logger, test tips successful. Lots of rust on the flat piece connecting wire to the long screws – maybe bring extras next time. One side of gauge clogged up (not touching bucket) cleared this out. Insecticide sprayed on wooden base and wires.

October 3, 2009

~11:45am

Gauge upright. Little debris in funnel. Swapped old data logger with new. Old data logger (NARHOUS) unresponsive. New battery in HOBO. Fastened gauge to new stake. No releveling necessary. Downloaded “NARHOUS” logger data successfully back at UNCA.

May 12, 2010

~ 1 pm (Light rain)

Gauge  upright, some debris in funnel, new battery in HOBO. Moderate re-leveling necessary; changed day from 13 May 2010 to 12 May 2010! Cleared nearby vegetation.

Directions:

Go to the overlook at mile marker 431 on Blue Ridge Parkway.

Follow the trail to the left of the picnic table (starts out paved). At the loop split, stay to the right.  When you pass the post marked #14, begin to look for fallen, cut tree, at the start of a descending portion of the trail.  Go to the right and catch a smaller, ranger trail marked by blue and red ribbons (very spread out).  When you see a small orange post, look in the clearing to the left for the gauge.