Gauge 111

Serial Number 746

Calibration Date: 2 June 2008

Installation Date: 17 July 2008

35˚43.781’ N, 82˚56.868’ W (~4573 ft)

Way Point 120

Visits:

17 July 2008 (9:06 AM)

Installation complete. Breezy, sunny.

5 August 2008 (1000)

Hazy, 2/8 cirrus cover

Data downloaded, date and time correct

Dead bugs cleaned out of gauge top (1 locust, 2 bees)

30 June 2009 (~1330)

Data download (Raw and accum). Gauge top was on, but had been knocked off by an animal (the holes for screws were torn open). Date/time correct. Data record stopped after 10/18/08, the logger will record tips on the WinComLog software when connected, but will not download these additional points. Cleared data, did test tips, downloaded successful. Cleared debris from gauge top and base. Only needed minor releveling. There was lots of dirt in the gauge, even beneath the Hydrological Services cover where the datalogger connects to the gauge. Gauge treated with insecticide and Bounce sheet added. Reattached gauge top and readjusted holes for screws using the hex wrench and wire clippers (hex wrench held straight across bent section of cover, pounded in with wire clippers).

18 July 2009 (~1600)

Data download (raw & accumulated). Cleared debris from gauge top and base. Some bird droppings in top. Some dirt in gauge, especially tried to clear out vents beneath bucket (no standing water). Full data record on logger. No releveling necessary. Added more insecticide. Put small washers on with cushions – this seemed effective at stopping gauge top movement when pushed from the side.

Directions:

40W to Exit 24 (209)

209N – Stay straight to get on Max Patch Road

Left on Wesley Creek Road. This road becomes gravel, keep following it. When you see the sign for Flying Hawk Way stay to your right. There is a split in the road, you need to go to the right through the gate. You need a key if you wish to drive through. There is a second gate which is held by a chain, not locked. Go through this gate and up the path to the right. Cross one meadow, go through another shorter section of woods, and it is up the hill on the second meadow. Use the GPS to help find it. This is the property of Fred Ferguson (452-4546).