Gauge 107

Serial # 743

Calibration Date: 5/27/08

Installation Date: 7/8/08

Lookout Point

35˚34.084’ N, 82˚54.449’ W, ~4459 ft

Way Point 111

Visits:

8 July 2008, 12:15 pm

Thunder in background, no rain by the time we left.

5 August 2008 1245

Both raw and accumulated uploaded (complete records)

Few insects in gauge

Gauge was off center, loosely connected to concrete. Took out pole from concrete and dug out hole to replace old concrete with new (full bag). Gauge leveled

4/8 sky cover with cirrus and small cumulus

30 June 2009 (~0830)

Downloaded data (raw and accum), did not clear. Cleaned out debris in gauge cover and gauge base. There were lots of cobwebs, dirt, and bird droppings. The gauge had to be tightened on the post as well as releveled (had to leave slightly off center in order to keep all the screws on). Dirt in one side of the gauge below the tipping bucket was clogging it, although no standing water was there (this clog was fixed). A Bounce sheet was taped in the gauge, post was treated with insecticide, and a new sign was affixed to the cover.

8 November 2009 (~0830)

Cleared debris from gauge top and inside gauge. Releveled, retightened on post. Downloaded data, did not clear. Had to adjust time by about 4 minutes (still in summer time) Sign and Bounce sheet intact. Weather: sunny and calm.

22 May 2010 (~7:30 am)

Gauge top pulled almost off – cow hair on gauge top and screws for base. Only one screw still connected the top, which was misshapen. Cleaned out gauge top, replaced Bounce sheet, cleared some cobwebs from near the data logger. Full data record, did not clear. Releveled. Date and time correct. Computer crashed during CSV-1 download but we were able to reboot and collect the rest of the data. Cleared some tall grass from the area. We banged out the gauge top so that it would fit again – it is still loose – you can’t just take it off, but it probably wouldn’t take too much effort from a cow. Wx – getting sunny, but some fog at gauge location. Rain earlier in the morning in the area.

12 June 2010 (~10:30 am)

Gauge needed releveling. Downloaded data again – full record. Water in tipping bucket tipped out during visit, it looked almost full. Added rubberized washers to gauge top as we did at Hurrican Ridge to try and make the top more secure. Cleaned out gauge top. Current wx – overcast. Low level fog in area. Some light rain beginning as we left.

Directions:

40W to Exit 27 (74W)

74W to Exit 106 (Clyde)

Merge on 19N/23N toward Clyde.

Left on Charles Street

Left on Hyder Mountain Road

Right on Chambers Mountain Road

Left on Lookout Point (follow this road to the right at the fork). You will come to a gate and need the key (we have this key) to open it.  This is John Terrell’s property: 627-2804(home), 550-1660(cell).