Gauge 102

Serial Number 754

Calibration Date: 5/23/08

Date Installed: 6/5/08, ~2:45 pm

Hemphill Bald at Cataloochee Ranch(~5365 ft)

35°33.823’ N, 83°06.214’ W

Way Point 58

Visits:

5 June 2008 (Installation ~1445)

Sunny, hot, some clouds.

4 August 2008 (~1300)

2/8 cumulus cover

Sign gone – dirt in gauge and top – cleaned out along with dead butterfly.

Gauge leveled, it was slightly off center

Data downloaded, date & time correct.

22 June 2009(~1445)

Cloudy. Dirt in gauge and top – cleaned out. One side of the gauge under the tipping bucket was clogged with dirt- not high enough to touch the bucket. This was cleaned out and then rinsed. Data downloaded, not cleared, date and time were correct. Gauge was releveled.

09 December 2009 (~1400EST)

Data downloaded; not cleared, date/time OK, little debris cleaned from 			funnel, gauge slightly re-leveled, clear skies & very windy!

19 May 2010 (12:14 pm EST)

Cloudy; slight releveling done. Applied Bounce sheet to gauge, cleaned gauge top. Downloaded data. Apparent bird remains on gauge. Changed time.

Directions:

See directions to Gauge 101 (TheSwag). Park in lot, and take the trail to your left, you will see signs to Hemphill Bald.  Our gauge is at the top of Hemphill Bald, just down the hill from the large stone table.