Gauge 307

Calibration Date: 5 May 2009

Installation Date: 10 June 2009, ~12:51 pm

Balsam Mountain Trail (35˚39.098’ N, 83˚11.971’ W, ~5327 ft)

Way Point 53

Visits:

10 June 2009 (Installation)

Mostly cloudy

24 October 2009 (~1100)

Tipping bucket about half full on arrival. Leaves and spider webs in top of gauge. Debris cleared from gauge top. Most leaves fallen from trees in area but some still remain. Slight releveling necessary. Bounce dryer sheet added and inside of gauge cleaned out. Data downloaded – full record, not cleared. Time adjusted to computer time – 4 minutes. Current weather: sunny, breezy.

5 June 2010 (~2:45 pm – partly cloudy)

Clean out debris, slight re-level, full data record (no clear), replace Bounce sheet, clean nearby vegetation, good time